### 1. Purpose

The purpose of this program is to establish a systematic and effective cleaning and sanitizing regime for all food contact surfaces, equipment, and facility structures. This program ensures that the environment is maintained in a hygienic state to prevent the survival and spread of foodborne pathogens, specifically addressing audit findings regarding improper drying and lack of a Master Cleaning Schedule.

### 2. Scope

This program applies to all areas of the establishment, including the kitchen, prep stations (Breading and Pasta), storage areas, restrooms, and dining areas. It covers all equipment such as deep fryers, slicers, small wares, and facility structures (floors, walls, vents, and grease traps).

### 3. Definitions

- Cleaning: The removal of soil, food residues, dirt, grease, or other objectionable matter.

- Sanitizing: The application of cumulative heat or chemicals on cleaned food-contact surfaces that, when evaluated for efficacy, yield a reduction of 99.999% of representative microorganisms of public health importance.

- Master Cleaning Schedule (MCS): A document that details what is to be cleaned, who is responsible, the frequency of cleaning, and the methods/chemicals used.

- Contact Time: The specific amount of time a sanitizer must remain wet on a surface to be effective.

### 4. Responsibility

- Lead Auditor/FSCO: Responsible for validating the MCS, verifying chemical concentrations via test strips, and conducting monthly sanitation audits.

- Store Manager: Responsible for enforcing the cleaning schedule, ensuring chemicals are available and labeled, and reviewing daily cleaning logs.

- Kitchen Staff: Responsible for performing the daily cleaning tasks according to the 5-step method and recording activity in the logs.

### 5. Procedure

#### 5.1 The 5-Step Cleaning & Sanitizing Method

To ensure effectiveness, all food-contact surfaces must follow these steps in order:

- Scrape/Pre-rinse: Remove loose food particles and heavy soil.

- Wash: Use hot water (110° F) and an approved detergent to break down grease and proteins.

- Rinse: Use clean water to remove all traces of detergent.

- Sanitize: Apply an approved chemical sanitizer at the correct concentration.

- Air-Dry: All items must be allowed to air-dry completely on clean racks. Prohibited Action: The use of drying towels is strictly forbidden.

#### 5.2 Master Cleaning Schedule (MCS)

The establishment shall maintain an MCS that identifies:

- Daily Tasks: High-touch surfaces (handles, switches), prep tables, slicers (every 4 hours), and floors.

- Weekly Tasks: Chiller interiors, shelving, and storage area perimeters.

- Monthly Tasks: Exhaust hoods/filters, ceiling vents, and deep-clean of fryers.

#### 5.3 Warewashing (3-Step Workaround)

Due to the structural limit of the 2-compartment sink, staff must:

- Use the 1st sink for Wash.

- Use the 2nd sink for Rinse.

- Use a dedicated, food-grade, sanitized plastic tub as the 3rd Basin for the Sanitize step.

#### 5.4 Sanitizer Solution Management

- Sanitizer Buckets: Dedicated buckets must be placed at the Breading and Pasta stations.

- Concentrations:

  - Chlorine: 50 - 100 ppm (Minimum 10 seconds contact time).

  - Quaternary Ammonium (Quat): 200 ppm (Minimum 30 seconds contact time).

- Verification: Staff must use approved test strips every 4 hours or whenever the solution becomes soiled to ensure potency.

#### 5.5 Grease Trap Maintenance

- 25% Rule: Grease traps must be inspected weekly. Cleaning must occur before the grease and solids layer reaches 25% of the total tank depth.

### 6. Monitoring

- Daily Cleaning Log: Supervisor verifies completion of all daily tasks at shift end.

- Sanitizer Log: Staff records test strip results for buckets and sinks every 4 hours.

- Visual Inspection: FSCO performs a "White Glove" check of vents and equipment during random audits.

### 7. Corrective Action

- Improper Sanitation: If a surface is found soiled or sanitizer is < 50 ppm, the item must be re-cleaned and re-sanitized immediately.

- Towel Use: If a staff member is found using a drying towel, the towel must be removed, and all affected items must be re-sanitized and left to air-dry.

- Grease Backup: If odors or slow drainage occur, an emergency grease trap service must be performed within 24 hours.

### 8. Verification

- Chemical Log Review: FSCO signs off on sanitizer and cleaning logs monthly.

- Environmental Swabbing (Optional): Quarterly swabbing of high-risk surfaces (Breading Station) to verify the absence of Salmonella.

### 9. Records

- Form LOG-CLN-01: Daily & Weekly Cleaning Checklist.

- Form LOG-SAN-01: Sanitizer Concentration Log.

- Form LOG-GRS-01: Grease Trap Maintenance Record.

- Form LOG-DEV-01: Deviation and Corrective Action Log.