### 1. Purpose

The purpose of this program is to ensure that the physical facility and all kitchen equipment are maintained in a state of repair that prevents food contamination and ensures the consistent performance of safety controls (e.g., temperature regulation). This program addresses structural integrity and equipment reliability as required by ISO 22000:2018 Clause 8.2 and RA 10611.

### 2. Scope

This program covers all physical structures (floors, walls, ceilings, doors), utilities (lighting, ventilation, drainage), and processing equipment (fryers, chillers, freezers, thermometers, and sinks) within the establishment and its commissary transport vehicles.

### 3. Definitions

- Preventative Maintenance (PM): Scheduled inspections and servicing to prevent equipment failure before it occurs.

- Calibration: The process of configuring an instrument (e.g., thermometer) to provide a result within an acceptable range of a known standard.

- Food-Grade Surface: Material that is smooth, non-absorbent, non-toxic, and easily cleanable (e.g., stainless steel, epoxy-sealed tiles).

- NIST Traceable: A calibration certificate proving the device was tested against standards held by the National Institute of Standards and Technology.

### 4. Responsibility

- Lead Auditor/FSCO: Responsible for verifying the calibration of thermometers, auditing the facility for structural defects, and reviewing maintenance logs.

- Store Manager: Responsible for scheduling repairs for structural issues (e.g., broken tiles) and ensuring all cooling units have functional thermometers.

- Kitchen Staff: Responsible for reporting equipment malfunctions and performing daily pre-operational checks on thermometer functionality.

### 5. Procedure

#### 5.1 Structural Integrity & Repairs

- Floors & Walls: Surfaces must be kept free of cracks and chips. Priority Action: Broken tiles must be repaired using food-grade grout and non-porous tiles to eliminate bacterial reservoirs.

- Exclusion Barriers: The hole in the back door must be sealed permanently with metal flashing or a heavy-duty door sweep to prevent pest entry.

- Sinks: While the current 2-compartment sink is a structural limitation, it must be maintained leak-free. A plan for 3-compartment installation must be developed for long-term compliance.

#### 5.2 Thermal Monitoring Equipment

- Cooling Units: Every refrigerator and freezer must have a permanent hanging thermometer positioned in the warmest part of the unit.

- Infrared (IR) Thermometers: An IR thermometer must be available on-site for rapid surface checks during receiving.

- Probe Thermometers: At least two 2 digital probe thermometers must be available and functional for "Kill Step" verification.

#### 5.3 Equipment Maintenance & Calibration

- Preventative Maintenance (PM): Fryers and cooling units must undergo a professional deep clean and mechanical check-up every six (6) months.

- Calibration: All probe thermometers must be calibrated weekly using the "Ice Slurry Method" (32°F / 0°C). Instruments that deviate by more than ± 2°F (±1°C) must be replaced.

#### 5.4 Utilities & Emergency SOPs

- Lighting: All bulbs in food zones must be shatterproof or shielded.

- Power Loss: In the event of a power outage, the establishment must follow the "Power Loss SOP," which includes keeping units closed and monitoring the 4-hour safety limit for TCS foods.

### 6. Monitoring

- Daily Pre-Op Check: Manager verifies that all chillers are at ≤ 40°F and freezers at ≤ 0°F.

- Weekly Calibration Check: FSCO performs and records the ice slurry test for all digital probes.

- Monthly Facility Walkthrough: FSCO inspects for new structural gaps, broken tiles, or lighting failures.

### 7. Corrective Action

- Temperature Failure: If a cooling unit is found above 41°F, food must be moved to a functional unit, and a technician called immediately.

- Calibration Failure: If a thermometer fails calibration, it must be "Red-Tagged" and removed from service. All batches measured with that device since the last "Pass" check must be evaluated for safety.

- Structural Repair: Temporary fixes for holes or broken surfaces must be applied within 24 hours, with permanent repairs scheduled within 14 days.

### 8. Verification

- Log Review: FSCO will sign off on the Maintenance and Calibration logs every visit.

- External Service: Invoices from professional repair contractors (HVAC, Plumbing) will be filed as verification of maintenance activity.

### 9. Records

- Form LOG-MAINT-01: Facility Maintenance & Repair Log.

- Form LOG-CAL-01: Thermometer Calibration Record.

- Form LOG-DEV-01: Deviation and Corrective Action Log.