### 1. Purpose

The purpose of this program is to ensure that all utilities (water, air, and energy) and waste management systems (solid and liquid) are designed and maintained to prevent contamination of food products. This program ensures the potability of water used in pasta preparation and the hygienic disposal of grease and kitchen waste as required by ISO 22000:2018 and P.D. 856.

### 2. Scope

This program applies to all utility inlets (potable water, electrical supply, ventilation) and waste outlets (grease traps, floor drains, trash disposal) within the establishment and the commissary transport link.

### 3. Definitions

- Potable Water: Water that is fit for human consumption and meets the microbiological and chemical standards of the PNSDW 2017.

- Backflow: The undesirable reversal of flow of water or other liquids into the potable water distribution system.

- 25% Rule: The requirement to clean a grease trap when the depth of floating grease and settled solids reaches one-quarter of the total liquid depth.

- Cross-Connection: Any physical connection between a potable water system and a non-potable source.

### 4. Responsibility

- Lead Auditor/FSCO: Responsible for coordinating annual water potability testing with DOH-accredited labs and auditing grease trap compliance. (If applicable)

- Store Manager: Responsible for monitoring the 25% rule for grease traps and implementing the "Emergency Utility SOP" during power or water failures.

- Kitchen Staff: Responsible for daily cleaning of floor drains and ensuring that hoses are never submerged in sinks or buckets (Backflow prevention).

### 5. Procedure

#### 5.1 Water Quality and Potability

- Testing: Water used for cooking pasta and making ice must be tested annually for microbiological and physio-chemical parameters (e.g., E. coli, pH, TDS) by a DOH-accredited laboratory.

- Ice Machines: Filters must be changed every 6 months or per manufacturer specifications to prevent biofilm buildup.

#### 5.2 Backflow Prevention

- Air Gaps: An air gap of at least twice the diameter of the water inlet must be maintained at all sinks.

- Mechanical Devices: Vacuum breakers must be installed on mop sinks and any hose bibbs. Hoses must never be left submerged in floor drains or buckets.

#### 5.3 Liquid Waste and Grease Management

- Grease Trap Monitoring: The thickness of the grease and sludge layers must be measured weekly using a dipstick.

- Cleaning Frequency: The trap must be cleaned before the waste layer reaches 25% of the total depth. Waste must be disposed of via licensed grease haulers.

- Floor Drains: Must be screens, cleaned, and flushed with hot water and sanitizer daily to prevent pest harborage.

#### 5.4 Solid Waste Management

- Bins: All trash cans must be leak-proof, fitted with tight-fitting lids, and emptied at least once per shift.

- External Area: The dumpster area must be paved and kept free of spilled grease or food debris to deter rodents and flies.

#### 5.5 Emergency Utility SOP

- Water Failure: If potable water is unavailable, food prep must cease. For temporary shortages, only bottled water from approved sources may be used.

- Power Failure: Refer to the Thermal Control PRP for the 4-hour safety window for TCS items.

### 6. Monitoring

- Weekly Grease Check: Manager measures the waste layer in the grease trap and records it in LOG-GRS-01.

- Daily Drain Check: Supervisor verifies that all floor drains are screened and free of odors.

- Annual Lab Report: FSCO verifies that a current water potability certificate is on file.

### 7. Corrective Action

- Grease Overflow: If grease is visible in the middle water layer or escaping the trap, the unit must be piped and cleaned within 24 hours.

- Backflow Risk: If a submerged hose is found, it must be removed immediately and the staff retrained on air-gap principles.

- Water Contamination: If a water test fails, the establishment must issue a "Boil Water" notice or use approved alternative sources until the system is disinfected and cleared.

### 8. Verification

- DOH Lab Review: FSCO reviews and signs the annual water analysis reports.

- Structural Audit: Monthly FSCO walkthrough to ensure no overhead dripping pipes or leaking grease traps.

### 9. Records

- Form LOG-GRS-01: Grease Trap Maintenance Record.

- Form LOG-UTIL-01: Utility Maintenance & Water Testing Log.

- Form LOG-DEV-01: Deviation and Corrective Action Log.