### 1. Purpose

The purpose of this procedure is to prevent foodborne illness by ensuring that all internal product temperatures are measured accurately. This SOP establishes standardized methods for the calibration and correct usage of thermometers to ensure that the poultry "Kill Step" and pasta cooling targets are reliably achieved, addressing critical findings.

### 2. Scope

This SOP applies to all food handlers involved in receiving, preparing, cooking, and cooling Time/Temperature Control for Safety (TCS) foods. It covers all temperature-measuring devices, including digital probe thermometers, infrared (IR) thermometers, and hanging unit thermometers.

### 3. Definitions

- Calibration: The process of checking and adjusting a thermometer's accuracy against a known standard (e.g., the freezing point of water).

- Ice Point Method: A calibration technique using an ice-water slurry to establish a 32° F (0 C) reference point.

- Kill Step: The cooking process designed to reduce pathogens to safe levels (standard: 165° F for poultry).

- Accuracy Window: The allowable deviation for a food thermometer, which must be within ± 2° F (± 1° C).

### 4. Responsibility

- Kitchen Staff: Responsible for sanitizing probes before each use, verifying batch temperatures, and performing weekly calibrations.

- Store Manager: Responsible for performing daily visual "Shadow Checks" of temperature probing and reviewing calibration logs.

- Lead Auditor/FSCO: Responsible for validating thermometer accuracy during randomized audits and training staff on the "Ice Point" technique.

### 5. Procedure

#### 5.1 Cleaning and Sanitizing

- Before and after every temperature check, the thermometer probe must be cleaned of food debris.

- Sanitize the probe using an alcohol-based wipe or a 100 ppm chlorine solution for at least 10 seconds.

- Store thermometers in a clean, protected area when not in use.

#### 5.2 Weekly Calibration (Ice Point Method)

- Preparation: Fill a large glass with crushed ice and add just enough cold potable water to reach the top of the ice. Stir the mixture well.

- Insertion: Submerge the thermometer probe at least 2 inches into the slurry, ensuring it does not touch the bottom or sides of the glass.

- Stability: Wait at least 30 seconds or until the reading stabilizes.

- The Standard: The reading must be 32° F (0° C).

- Frequency: Perform this check every Monday before the opening shift, or immediately if the thermometer is dropped or exposed to extreme temperatures.

#### 5.3 Correct Probing Technique

- Placement: Insert the probe into the thickest part (geometric center) of the food item. For bone-in chicken thighs, avoid contact with the bone, as this will provide an inaccurate reading.

- Holding Time: Maintain the probe in the food until the reading remains steady for at least 15 seconds for "Kill Step" verification.

- Packaged Goods: For receiving, place the probe between two packages without puncturing the primary packaging.

### 6. Monitoring

- Staff must record all calibration results in LOG-CAL-01.

- All cooking/cooling batch temperatures must be recorded in their respective logs (LOG-COOK-01 or LOG-COOL-02).

### 7. Corrective Action

- Calibration Failure: If a device deviates by more than ± 2° F (± 1° C):

  - Bimetallic Dial: Adjust the calibration nut under the head until it reads 32° F.

  - Digital: Reset according to manufacturer's instructions. If it cannot be adjusted, "Red-Tag" the device and discard it.

- Safety Assessment: If a thermometer is found to be inaccurate, all food batches measured with that device since the last "Pass" result must be evaluated and discarded if safety cannot be guaranteed.

### 8. Verification

- The Store Manager will initial the Calibration Log daily to verify compliance.

- The FSCO will perform an independent "Ice Point" check of all on-site thermometers once per month.

### 9. Records

- Form LOG-CAL-01: Thermometer Calibration Record.

- Form LOG-COOK-01: Batch Cooking & Reheating Log.

- Form LOG-COOL-02: Pasta Cooling & Reheating Log.

- Form LOG-DEV-01: Deviation and Corrective Action Log.